# Low-Precision Transformer Accelerator for Edge LLMs

# Overview / Summary

This project proposes the design and implementation of a low-precision hardware accelerator for Transformer-based language models on a Xilinx Kria KV260 or AUP-ZU3 FPGA platform. The goal is to accelerate quantized inference for a reduced Transformer block (TinyLlama-scale) while addressing memory bandwidth limitations (“memory wall”) common in edge AI systems.

The system uses a heterogeneous architecture: the ARM processor handles tokenization, scheduling, and baseline execution, while the FPGA fabric accelerates matrix multiplications and normalization using a custom INT8 compute engine with DDR streaming and double buffering. Phase 2 explores mixed-precision INT4 weights and limited attention support.

# Block Diagram

# Objective / Motivation

Transformer inference is dominated by matrix multiplication and memory movement. On edge platforms, memory bandwidth is often the primary bottleneck. This project explores compute–memory co-design for quantized Transformer acceleration.

Objectives:

- Design an INT8 matrix multiplication accelerator.

- Implement DDR streaming with double buffering.

- Measure latency and resource utilization.

- Explore INT4 weight compression to reduce bandwidth.

The focus is on accelerating key components of a Transformer block (linear layers and feed-forward network) rather than full multi-layer LLM deployment.

# Plan / Methods

The project will proceed in two structured phases:

## Phase 1

- Quantize a reduced Transformer block to INT8.

- Implement INT8 MAC array with INT32 accumulation.

- Store weights in DDR4 and stream via AXI-DMA.

- Implement double buffering in BRAM.

- Accelerate feed-forward network (two linear layers + activation).

- Compare FPGA vs ARM CPU latency.

- Report LUT, DSP, BRAM, and Fmax.

## Phase 2

- Support packed INT4 weights.

- Implement limited single-head attention with approximate softmax.

- Add bounded KV cache support.

- Evaluate memory bandwidth impact.

# Software / Hardware

### Hardware Platform

- Xilinx Kria KV260 or AUP-ZU3 (Zynq UltraScale+)

### Development Tools

- Vivado / Vitis

- Verilog RTL (primary hardware description)

- AXI-DMA IP cores

- Python (model quantization, golden reference)

- C/C++ (ARM baseline implementation)

### Software Components

- Tokenization and scheduling on ARM

- CPU baseline Transformer block in C++

- Host-side performance measurement tools

# Expected Outcome

By the end of the semester, we expect to demonstrate:

- A working INT8 Transformer component accelerator.

- Correct results validated against a software baseline.

- Measurable latency improvement over ARM CPU execution.

- Detailed resource utilization report.

- Analysis of compute vs memory bottlenecks.

If Phase 2 is partially completed, we expect to demonstrate bandwidth reduction from mixed precision and limited attention acceleration.